Guide Specification

PART 1 – GENERAL			      				             09 50 00 - Ceilings

  - SUMMARY

- All labor to fabricate and deliver the concealed suspended ceiling system along with acoustical “Grooved Wood Planks”, trims and required accessories

  - RELATED WORK IN OTHER SECTIONS

- Finished partitions, wall, and other ceiling structural elements

- Integrated assembly including but not limited to lighting fixtures, speakers, electrical cables, and related connections

- Mechanical and/or plumbing services, sprinklers, smoke detectors HVAC systems and related accessories

- Surface paint finishes and any other acoustical elements outside of this scope

  - REFERENCES

- ASTM C423: Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method

- ASTM E84: Standard Test Method for Surface Burning Characteristics of Building Materials

- CAN/ULC-S102-10: Standard Method of Test for Surface Burning Characteristics of building Materials and Assemblies

- U.S. Green Building Council, LEED Building Design and Construction, Version 4.0

- CISCA: Ceiling Systems Handbook

  - MANUFACTURER QUALITY ASSURANCE

- Manufacturer must have the capacity to manufacture the specified product to meet the qualified defined standard and quality for a period of not less than five years

- Any product substitution request including the overall design, product performance, dimensional changes, mounting and finishes must be submitted to all stakeholders including the architect within 10 working days prior to the latest proposal due date

- Dimensional tolerances and size variation of nominal 1/8” (3mm) will be expected due to the nature of wood products

- Planks must have anti-microbial finish

- The manufacturer must be able to certify the wood products with FSC standard, principles, and criteria for forest stewardship with chain of custody certification

- Planks must be independently certified to conform to California Department of Public health (CDPH) Standard Method v1.2: Private Office and School Classroom.

- Wood wall and ceiling mounted planks must be tested in accordance with ASTM E84 surface burning characteristics by a third-party acceptable testing facility as a finished composite material with Class A (FS=25 or less – SD=450 or less) performance

- All specified wood wall and ceiling planks with acoustical absorption characteristics must be tested in accordance with ASTM C423 and ASTM E795 as indicated by an authorized testing organization acceptable to authorities having jurisdiction. Approved testing organization must be independent of the manufacturer

- The qualification, competency, and capability of an experienced installer(s) with a successful service record should be approved by the manufacturer prior to the installation

  - SUBMITTALS

- A complete set of shop drawing(s), RCP, layout, product details and installation instructions are required, showing all necessary dimensions, penetrations, interferences, discrepancies, notes for the architect approval.  Must be subsequently field verified by the installers

- Technical data sheet, acoustical and fire performance test reports for the specified product to comply with the required criteria

- LEED documentations including recycle content, CDPH low emitting materials certification, and distances

- Three (3) sets of samples for each finished product to be manufactured as original material for verification of each type of exposed finish required, prepared on sample of size 8” × 11”

- Acoustical and Fire test reports on supplied product(s) as components to meet the specified requirements issued by a third party certified independent laboratory

  - DELIVERY, STORAGE, AND HANDLING

- All crate sizes, weights, unloading options, scheduled delivery and unloading of suspension system in original unopened packages must be coordinated with the single source manufacturer prior to fabrication to be stored in fully enclosed on-site acclimatized storage, protected against damage from moisture, direct sunlight, and other mistreatment

- Installation will only occur after materials have been acclimatized for a minimum of 72 hours in climate-controlled site condition between 60-90 degrees Fahrenheit (15.5-32 degrees Celsius) inclusive that meet the optimum moisture content and relative humidity requirements of 25% to 55% as per North American Architectural Woodwork Standards.

- Wood planks must be handled and installed by persons wearing clean light-weight gloves

  - WARRANTIES

- A reasonable percentage of extra wood planks and clips shall be furnished if specified with protective covering for storage

- Manufacturer shall provide owner with a (1) year warranty against any manufacturing defect excluding abnormal site conditions, wood stain discoloring due to the age for all the installed material described in this specification from the final acceptance of completed work

PART 2 – PRODUCTS

- Basis of Design: MCM Acoustical.  6860 Rexwood Road, Mississauga, ON  L4V 1L8  Canada.  info@mcmacoustical.com 905-670-3733

- Product: GROOVED WOOD PLANKS with nominal thickness of 16mm (5/8”).  Nominal width 192mm (7 9/16”).  Nominal length 8’ (2.4m) and 10’ (3m)

- Veneer Option: Quarter cut, Slip Match, Cut, etc.

- Plank Construction: Natural wood veneer to be laminated on fire rated medium density fiberboard (MDF) and grooved.  Back and core of the plank to be have 9mm perforations staggered based on face groove spacing. The MDF core material shall be furnished with special acoustical black matt at the back.  Sides of planks to be tongue and grooved

- Suspension System: Planks are installed progressively with provided clips that screw into grid or blocking.  Standard grid and all other suspension hardware including the hanger wires, rods, braces, and moldings to be provided by others as per ASTM standards

PART 3 – EXECUTION

  - PREPERATION AND INSTALLATION

- Installation shall not begin until all substrates and the ceiling layout properly prepared to conform with reflected ceiling plans and approved shop drawings

- It is the contractor’s responsibility to provide required support to install suspension system without interference with other elements and perimeters

- The workspace must be fully enclosed to maintain the ambient environment as per section 1.6 of this specification.

- The suspension system installation and engaging the wood planks must comply with all local codes and regulations, manufacturer's installation instructions